Crawford VL3010 vertical lifting fabric door

Product description:

- Vertical lifting fabric door designed for industrial environments where doors are exposed to moisture, dust, very high or low temperatures or where the door opening is large.

- The unique design and structure offers durability, tightness, energy efficiency, operational reliability and minimum maintenance.

Door leaf

- Door leaf made out of two layers of very strong vinyl-coated polyester fabric, separated by aluminium intermediate sections.

- Intermediate section which gives strength to the door leaf and which creates a buffer between the inside and outside fabric walls.

- Wind load is transferred to the vertical guide rails by the horizontal aluminium sections of the door leaf.

- The door leaf is available in 8 standard colours.

- Door leaf thickness:100 mm.

- Aluminium guide rails.

Bottom section

- The bottom section, made of steel and aluminium, is connected to the lifting belt via the safety arresters

- The bottom section contains an optical safety edge and a rubber seal on the bottom edge that creates a seal between the door and the floor.

- Optical safety edge.

Safety arresters and wind lock in stainless steel

- Safety arresters stop the door if the lifting belt becomes slack or, in an unlikely event, would break.

- Wind lock system locks the door when the door is closed.

Surface treatment

- Steel components corrosion treatment: category 3 according to ISO 12944.2. Higher class on request. Aluminium, plastic, stainless steel, zinc electroplated steel (~10u). Fixing elements are mainly hot dip galvanized (FZV). Door leaf screws protected with Geomet.

Standard door features

- Door range:

MIN: 1810mm x (no MIN. height) MAX: 8000 x 12000 mm

- Normal operating speed of 0.2-0.3 m/sec.

- Option: Increased opening speed of 0.4-0.6 m/sec.

- Temperature range: .-35 °C to +70 °C

- Compliance with all operational and safety instructions in EU Directives and the standards of the European Standardisation Committee (CEN).

- Wind load (EN 12424)	Class 3-5, 0.7-1.6 kPa, depending on

size

- Water resistance (EN 13241)	Class 3, 0.11 kPa (for a closed door)

- Air permeability (EN 13241)	Class 2, 12 m3/(m2h)

- Thermal transmittance (EN

12428)

3.4 W / (m² K)

- Sound reduction (ISO 717)	15 dB Rw

- Operating system: Electrical operator.

Dimensions:

x	mm (WxH)

Options:

Door leaf:

  - Standard: Polyester, 1100 dtex with plasticised PVC coating

  - Option: Arctic fabric. For environmental temperatures down to -45°C

  - Option: Heat resistant fabric with silicone coating

  - Option: Heat resistant fabric with aluminium coating

  - Option: Heat resistant fabric with polyurethane coating

  - Option: Sound reduction fabric

  - Option: Security fabric

Vision panels

800mm x 800mm

800mm x 1600mm

1300mm x 800mm

1300mm x 1600mm

Fabric color

  - Tan (RAL 1001)

  - Red (RAL 3001)

  - Blue (RAL 5005)

  - Green (RAL 6009)

  - Grey (RAL 7004)

  - Anthracite (RAL 7016)

  - White (RAL 9016)

  - Translucent white

Header box

  - Gear motor on inside of the building

  - Gear motor on outside of the building

  - Gear motor in the door opening

  - Insulated

  - Stainless steel cladding and motor casing

  - Steel cover for limit-switch box

  - Dust-proof limit-switch box

  - Heater in limit-switch box

  - Self-supporting header box

Operator

- Control system: PLC-based

- Protection class, control cabinet IP65

- Protection class, limit switches IP65

- Protection class, motor brake IP55

- Protection class, push buttons IP65

- 3/phase 400V 50H power supply

- Control voltage 24V AC

- Fusing: 16 A

- Free contacts: 6 for control of user functions

- Heat and cold resistance: -35 °C to +70 °C

- Motor ratings: 0.65 - 3.7 kW

Options

  - Radio remote control

  - “Open" (during closing process) if open button is used - door immediately stops and opens completely)

  - Pull switch for OPEN / CLOSE

  - External push-button, key switch

  - “One-button function” - opens or closes door, depending on actual status (e.g. radio, pull switch)

  - Additional safety (e.g. photo cell, radar) – door immediately opens completely

  - Photo cell

  - Reduced opening height

  - Radar

  - Magnetic loop

  - Warning lights

  - Traffic lights

  - Interlocking door function

Other supplementary equipment

  - Control cabinet in stainless steel

  - Heating element in the control cabinet

  - Emergency power switch with separate steel casing

  - Clamp strip covers in the same colors as the fabric

  - Side jambs (if posts for installing the guide rails are not available)

  - Side jambs in stainless steel

- Wind deflectors (in combination with jambs)